N2O / Paraffin Hybrid

Injector Sizing Tool

Validation Report

What has been verified, what has not, and what it would take

Generated 26 September 2026

Contents

Right-click here and choose “Update Field” to build the contents.

# Validation status, roadmap, and known limitations

Read this before using the tool for hardware decisions.

Bottom line: the injector flow models have now been checked against real measured N2O cold-flow data (§1.4) and land where theory says they should. The tool has still never been compared against a static fire or a real HRAP run, and the motor side — regression, blowdown, chamber — remains unvalidated against hardware. Trust the injector models to a few percent for orifices similar to the tested ones; do not yet trust predicted burn time, O/F history, or impulse without your own firing data.

## 1. What was built, and how it was verified

### Workflow followed

Read HRAP's actual source rather than assuming its behaviour — the MATLAB util/*.m, the Python hrap/*.py, the HRAP.mlapp GUI internals, and the theory PDF that ships in the repo. This established: HRAP's injector is SPI only; its N2O properties are ESDU 91022; its CEA tables come from RPA; its regression law uses oxidiser flux; and its exact unit and state-flag conventions.

Implemented the fluid properties first and cross-validated them against an independent reference EOS before building anything on top.

Implemented the three flow models, then checked each against the physical limits it must satisfy rather than against expected output values.

Built the burn simulation to HRAP's formulation so that SPI mode is a like-for-like reproduction.

Built the sizing solver on top, then the GUI, then HRAP import.

Every stage got tests before moving on; two genuine physics bugs and two robustness bugs were caught this way (listed in §1.3).

### Testing performed — 102 tests, all passing

python -m pytest tests/ -q

| Group | What is actually checked |
| --- | --- |
| Fluid properties | ESDU vs published saturated N2O at 20 °C (P, ρ_l, ρ_v to 1–2 %); critical point exact; monotonicity of the saturation curve; Tsat inverts Psat; latent heat decreases toward critical; s_fg = h_fg/T exactly |
| Cross-backend | ESDU curve fits vs CoolProp's reference EOS over −20…+30 °C: pressure 0.1 %, ρ_l <1 %, ρ_v <3 %, hfg <2 %, sfg 0.4 %. HEM mass flux agrees to 0.05 % between the two |
| Flow-model limits | SPI is exactly √(2ρΔP); SPI > Dyer > HEM for a saturated feed; κ = 1 when saturated, making Dyer the exact mean; Dyer → SPI when the feed cannot flash; more subcooling raises κ; L/D → 0 gives SPI and large L/D gives HEM; HEM chokes and plateaus; critical pressure ratio 0.72; no flow when downstream ≥ upstream; flux monotone in chamber pressure; fast precomputed curve matches the direct solver to 0.2 % |
| HRAP parity | SPI reproduces HRAP's dLoss = K/(NA)², ṁ = √(2ρΔP/dLoss) to machine precision; liquid-mass split, regression law and c* formula match HRAP's; constant-O/F matches const_OF.m |
| HRAP import | Unit factors match HRAP's initialiser (incl. its psi = 101325/14.696); imperial and metric; both tank-volume modes; tank state as temperature or pressure; fill as mass or percentage; nozzle as ratio or exit diameter; C/K/F/R round-trip; export→import round-trip; propellant .mat with transposed CEA grids |
| HRAP CSV | Unit conversion; 13- and 15-column variants; recovery of the true injector ΔP; rejects non-HRAP files |
| Comparison | Identical runs → ~0 % error; an injected 10 % perturbation shows up as one |
| Conservation | Integrated oxidiser and fuel flow match tank/grain depletion to 2 %; port opens monotonically; tank stays on the saturation curve; chamber always below tank |
| Failure modes | Liquid-full and vapour-only tanks, out-of-range temperature, ports that don't fit, sub-minimum holes, n ≥ 1, degenerate designs |

Two internal consistency results worth noting, because they were not designed in and could have failed:

The two chamber models converge — quasi-steady and HRAP's transient ODE end at 32.04 vs 32.06 bar with total impulse within 3 %.

The two property backends agree on HEM flux to 0.05 %, even though ESDU carries no entropy data and the entropy had to be reconstructed thermodynamically. That is the strongest evidence the HEM branch is right.

### 1.4 External validation against measured NASA/Stanford data

Source: B. S. Waxman, J. E. Zimmerman, B. J. Cantwell (Stanford) and G. G. Zilliac (NASA Ames), Mass Flow Rate and Isolation Characteristics of Injectors for Use with Self-Pressurizing Oxidizers in Hybrid Rockets, NTRS 20190001326. Reproduce with:

python examples/validate_waxman.py

The paper implements the same three models and its equations confirm the implementations line-for-line — SPI (Eqn. 2), HEM (Eqns. 3–5), and Dyer/NHNE with κ = √((P₁−P₂)/(Pᵥ−P₂)) (Eqns. 8–9), identical to what is coded here.

Test case — injector 3: straight hole, D = 1.50 mm, L = 18.4 mm (L/D = 12.3), rounded inlet, N2O at P₁ = 704 psia, T₁ = 280 K, supercharged 169 psi, measured single-phase Cd = 0.77.

| Check | Paper | This tool | Result |
| --- | --- | --- | --- |
| Supercharge (pins Psat via REFPROP) | 169 psi | 166 psi | −2.0 %, implying Psat agrees to 0.63 % |
| SPI valid up to | ΔP ≈ 200 psi | onset at ΔP = 166 psi | correct direction — real flow flashes late (metastable) |
| Cdeff at ΔP = 330 psi | ≈ 0.60 | HEM 0.545 / Dyer 0.677 / SPI 0.770 | HEM −9.1 %, Dyer +12.8 %, SPI +28.3 % |
| Critical (choked) flow observed | yes | yes | ✓ |

What this establishes:

The measurement is bracketed by SPI above and HEM below, which is the ordering the physics demands. A result outside that bracket would have meant a coding error.

HEM under-predicts by 9 % — and it must. The paper states equilibrium "gives a lower-bound estimate for the critical mass flow rate" because real flashing is delayed by metastable superheat. Landing just below the data is the correct behaviour, not a defect.

SPI over-predicts by 28 %. This is the paper's central practical point, and it is exactly why HRAP's SPI-only liquid model will over-predict oxidiser flow for a flashing injector.

The tool's own L/D guidance is confirmed by the data. It recommends HEM for L/D > 10; at L/D = 12.3, HEM is indeed the closest model. That guidance was written from the literature before this comparison was run.

Limits of this check. The reference numbers are quoted from the paper's prose and figure captions, not a data table (the full dataset is in an appendix of an earlier paper by the same authors, not to hand), and Cdeff is stated as "approximately 0.6". So this is good to a few percent, not three digits. It covers one injector at one operating point; the paper spans 500+ tests across five injectors, D = 0.79–1.93 mm, L/D = 9.5–23.4, and supercharges from 41 to 500 psi. Every tested orifice was long (L/D ≥ 9.5) — there is still no external check in the thin-plate regime (L/D < 5) where Dyer is the recommended model and most practical injector plates actually sit.

Locked in as 11 tests in tests/test_external_validation.py.

### 1.5 Bugs found and fixed during development

These are listed because they indicate the class of error that testing catches — and, by implication, what might still be lurking in untested areas.

HEM returned zero flux above ~30 bar. h₁ − h₂ in a flashing expansion is a few kJ/kg out of ~50, so independently-fitted enthalpy and entropy disagreed by more than the signal. Fixed by deriving entropy from the ESDU enthalpy fit via T ds = dh − v dP.

Choked plateau wobbled because the scalar HEM solver anchored its pressure grid at P₂. The plateau is physically flat; now the grid is anchored independently of P₂.

Transient chamber never settled — chamber gas mass was seeded at ~0 instead of HRAP's 1.225 × free volume, stretching the ignition transient across the whole burn.

An infeasible design-point guess aborted objectives that never use it, and a target that drives ΔP to ~0 returned a converged-looking but meaningless area. Both now handled explicitly.

## 2. What to do to actually trust it

Ordered by value per unit effort.

### Tier A — physical testing (only you can do this)

A1. Cold-flow the injector plate. Highest value single action. Water first: cheap, safe, and gives you the geometric discharge coefficient directly (Cd = ṁ / (A√(2ρΔP))). Then N2O if you can do it safely: this is the only way to tell which of SPI/HEM/Dyer describes your actual plate. Sweep several ΔP values and, if possible, two feed temperatures. Send me the ṁ / ΔP / T data and I will fit Cd, overlay all three models, and tell you which one your hardware follows.

Cd = mA 2ρΔP

A2. Static fire with instrumentation. Minimum useful set: tank pressure, tank temperature, chamber pressure, thrust, and propellant masses before and after. Time-align everything to a common trigger.

A3. Measure your own regression coefficients. Pre- and post-fire port diameter and grain mass. Two firings at meaningfully different oxidiser flux give you a and n by fitting ṙ = a·Gox^n. See §3.1 — this matters more than everything else combined.

A4. Weigh the N2O load and the residual. Blowdown ends with a significant vapour residual; confirming it validates the tank model independently of combustion.

### Tier B — analysis I can do now, with no new data

B1. Timestep convergence study. Confirm results are converged at your dt and report the error at each. The integrator is forward Euler (matching HRAP); this is currently unquantified and is the most likely source of silent error.

B2. Monte Carlo uncertainty propagation. Give me your input uncertainties (±Cd, ±a, ±n, ±fill mass, ±temperature) and I'll return a distribution on hole diameter, O/F and impulse instead of a single number. Given §3.1 this is probably the most useful thing I can produce without new data.

B3. Lock in a reference-case regression test so future changes cannot silently move the answers.

B4. Grid-independence check on the HEM solver (currently 600 points) and on the saturation table.

~~B5. Find and digest published N2O injector data.~~ Done — see §1.4. The remaining gap is the thin-plate regime (L/D < 5), which the NASA dataset does not cover. Worth hunting for a second dataset there, since that is where most practical injector plates sit and where Dyer is the recommended model.

### Tier C — needs one thing from you, then I can finish

C1. Real HRAP cross-check. Export a CSV from your HRAP install for a motor you care about, and I'll run the comparison and chase any discrepancy. This is now a 3-click workflow in the GUI. This is the obvious next step.

C2. Tailored sensitivity study on your actual motor rather than my demo.

### Tier D — missing physics I can implement on request

Listed in §3.3 with the size of the error each one is hiding.

## 3. Notes, warnings, and known inaccuracies

### 3.1 The regression coefficients — what the sensitivity actually means

Correction to an earlier version of this document. The sensitivity table below is a property of the inverse problem this tool solves, not evidence that HRAP's coefficients are wrong. HRAP runs forwards — fixed geometry, predict O/F — and in that direction the same coefficient errors are benign:

| +10 % error | Forward (HRAP's direction): resulting O/F | Inverse (sizing to an O/F target): resulting area |
| --- | --- | --- |
| coefficient a | −8.7 % | +148 % |
| exponent n | −33 % | +10456 % |

The amplification is created by the inversion. To hit a target O/F you must solve ṁox ∝ [ … ]^(1/(1−n)); for paraffin n ≈ 0.68, so that exponent is 3.13 and a 10 % error in a becomes 35 % in required flow. A second amplification follows: a larger injector raises chamber pressure, which cuts the injector ΔP, which demands more area again. HRAP never does this inversion, so this was never an HRAP problem.

mox ∝  ⋯ 11 − n

This is now fixed. Two objectives were added that size the orifice from the feed state alone and never touch the regression law:

| Objective | a +10 % | n +10 % |
| --- | --- | --- |
| burn_average (O/F target) | +148 % | +10456 % |
| `mdotox` (target oxidiser flow) | +1.0 % | +1.8 % |
| `chamber_pressure` (target Pc) | −1.4 % | −2.2 % |

The physical design barely changes — sizing the demo motor for 1.53 kg/s gives a 1.697 mm hole against 1.699 mm from the O/F objective. You get the same injector, arrived at in a way that is roughly 5000× less sensitive to coefficient error. The tool now emits an ILL-CONDITIONED OBJECTIVE warning, and reports 1/(1−n), whenever an O/F objective is used.

Recommended workflow: size with mdot_ox or chamber_pressure, then run forwards and read off the O/F you actually get. Use the O/F objectives only once a and n are measured on your own motor.

### 3.1b Why the coefficients still can't be transplanted

None of the above means the shipped coefficients are usable as-is. HRAP's own theory document is explicit (§II.B, eq. 11):

the values a, n and m are experimentally determined constants which depend on the propellant being used, and the oxidizer injection system. Injection systems which induce a vortex in the oxidizer flow substantially increase regression rates compared to simple axial injection…

So a and n are not a fuel property. They encode the fuel and the injector and the test article, fitted over one flux range. HRAP also says the constant-O/F model exists precisely "in cases where ballistic coefficients for the propellant are unknown", to be replaced "once sufficient testing is conducted" — which is why every motor config HRAP ships uses constant-O/F rather than the shipped coefficient sets.

A concrete illustration, using only coefficients read from HRAP's own files:

| ṙ [mm/s] at G = | 50 | 100 | 200 | 350 | 500 |
| --- | --- | --- | --- | --- | --- |
| HTPB (a=0.198, n=0.325) | 0.71 | 0.88 | 1.11 | 1.33 | 1.49 |
| 50/50 HTPB-paraffin (a=0.1146, n=0.5036) | 0.82 | 1.17 | 1.65 | 2.19 | 2.62 |
| Paraffin (a=0.0304, n=0.681) | 0.44 | 0.70 | 1.12 | 1.64 | 2.09 |

Pure paraffin comes out slower than the 50/50 blend across the entire range (ratio 0.53 → 0.80), and no faster than plain HTPB below G ≈ 200. Taken at face value that is backwards. The likely explanation is not an error in HRAP but exactly what its documentation warns about: these entries come from different motors with different injectors and different fitted flux ranges, so they are not comparable to each other and none of them is a generic "paraffin" curve.

Comparing `a` values across different `n` values is meaningless anyway — only ṙ at a stated G is comparable. That is the correct way to sanity-check any coefficient set you are handed, including your own.

### 3.1c Sensitivity table (inverse problem, O/F objective)

Measured sensitivity of the sized injector area to a +10 % change in each input, for the demo motor (24 holes, Dyer, target O/F 8.27):

| Input +10 % | Sized Cd·A | Hole diameter | Leverage |
| --- | --- | --- | --- |
| Regression exponent n | +10456 % | +927 % | 1046× |
| Fuel density ρ | +201 % | +74 % | 20× |
| Grain length | +201 % | +74 % | 20× |
| Target O/F | +157 % | +60 % | 16× |
| Regression coefficient a | +148 % | +58 % | 15× |
| Initial port diameter | −10.6 % | −5.5 % | 1.1× |
| Throat diameter | −10.4 % | −5.3 % | 1.0× |
| Tank temperature (+5 °C) | −10.0 % | −5.1 % | 1.0× |
| c\* efficiency | +4.1 % | +2.0 % | 0.4× |
| Discharge coefficient Cd | 0 % | −4.7 % | — |
| Flow model: SPI instead of Dyer | −21 % | −11 % | — |
| Flow model: HEM instead of Dyer | +37 % | +17 % | — |

This table applies only to the O/F-based objectives. With mdot_ox or chamber_pressure the top five rows collapse to a couple of percent (§3.1).

Practical consequences:

Use a flow-based objective unless you measured `a` and `n` yourself. That removes the amplification entirely and costs nothing in design quality.

HRAP's shipped paraffin values are a starting point, not your motor — see §3.1b. Measure your own; they depend on your injector as well as your fuel.

Cd does not change the required effective area Cd·A (that is what gets solved for) — it changes the hole diameter you must drill to achieve it.

Once a flow-based objective is used, the injector flow model becomes the dominant modelling choice again (SPI −21 %, HEM +37 % on area), which is the question this tool was built to answer.

### 3.2 Warnings about interpreting results

The `Dyer (L/D weighted)` model is a heuristic I wrote, not a published model. It exists to expose the regime trend and to make the L/D limits testable. Do not use it for design. Use Dyer.

A converged answer is not necessarily a sensible one. If the target is unreachable, the solver drives chamber pressure onto tank pressure and returns a huge area with near-zero ΔP. It now flags this as DEGENERATE RESULT, but check the ΔP margin on every run.

`Cd = 0.7` is a placeholder, not a prediction. It is a direct multiplier on mass flow. Measure it (A1).

Constant-O/F mode cannot size an injector — O/F is pinned by definition, so any area satisfies it. The tool refuses rather than pretending. Use it only to reproduce an HRAP run.

ESDU correlations are valid −90 °C to +36 °C. N2O's critical point is 36.4 °C. A hot day on the pad genuinely approaches this, where properties move fast and the model stops. The tool errors rather than extrapolating.

Burn ends at liquid depletion by default. The vapour-phase tail is modelled but not included unless you ask for it. Real impulse includes some of that tail.

The dP > 15–20 % rule is a rule of thumb, not a stability model. Meeting it does not guarantee stability; missing it does not guarantee instability. There is no combustion-instability model in this tool.

### 3.3 Physics not modelled — and how much it could cost you

| Not modelled | Likely effect | Can I add it? |
| --- | --- | --- |
| Feed system between tank and injector (plumbing, valve, bends, manifold) — upstream state is taken at the tank | Over-predicts ΔP across the injector, hence over-predicts flow. Can be several bar on a long/narrow run | Yes — needs your line sizes and Cv |
| Vents (HRAP has None/External/Internal) | Oxidiser flow and blowdown rate both differ. Flagged on import, not simulated | Yes, straightforward |
| Cd variation with Reynolds number, cavitation, and inlet geometry | Cd is constant; real Cd shifts through the burn | Partly — correlations exist but are geometry-specific |
| Injector element type (showerhead / impinging / swirl) | Affects both Cd and mixing efficiency; the latter feeds c\* efficiency | Only empirically, from your data |
| Tank heat transfer — blowdown is adiabatic, with liquid and vapour in thermal equilibrium | Real tanks draw heat from walls, so pressure decays slower than predicted. Predicted burn time is likely conservative. HRAP shares this assumption | Yes, with a wall model |
| Non-equilibrium bubble dynamics | Dyer's κ is a correlation, not physics. Real flashing depends on nucleation-site density and residence time | Only with a much heavier model |
| Paraffin entrainment — the liquid-layer physics that makes paraffin fast-regressing | The a·G^n power law is empirical and may not extrapolate outside the flux range it was fitted in | No — this is why you must measure a and n |
| Throat erosion | Chamber pressure decays over a long burn; impulse over-predicted | Yes, with an erosion rate |
| Grain end-face regression | Slightly under-predicts fuel flow. HRAP explicitly assumes this too | Yes |
| Complex port geometry — only N identical circular ports | Multi-port web is a nominal equivalent-area figure, not true bolt-circle geometry. HRAP's Python build does arbitrary cross-sections | Yes, with effort |
| c\* efficiency as a constant | Real η_c\ varies with L\, O/F and port geometry | Yes, as a curve |
| Ignition transient / igniter mass | First ~100 ms of the transient run is not meaningful | Yes |
| Combustion instability | No model at all; only the static ΔP heuristic | No — outside scope |
| Mass properties / CG | Not computed. HRAP does this | Yes, easily |
| Nozzle losses beyond a lumped efficiency (divergence, boundary layer, two-phase) | Absorbed into noz_eff, so thrust is only as good as that guess | Partly |

### 3.4 Numerical caveats

Forward Euler integration (matching HRAP), so accuracy is first-order in dt. This is not yet quantified — see B1. Default dt is 5 ms for sizing and whatever the HRAP config specifies on import (often 1 ms).

Quasi-steady vs transient chamber differ by ~3 % on impulse in the case I checked. Quasi-steady is the default for sizing (robust, no ignition transient); transient is for HRAP comparison.

The HEM solver uses a 600-point pressure grid with local refinement, and saturation properties come from a 3000-point interpolated table. Both are far below the correlation's own uncertainty, but neither has a formal convergence study.

The saturation table caches on temperature quantised to 1 mK.

## 4. Suggested order of work

C1 — export one HRAP CSV and run the comparison. Cheapest possible external check, and it either builds confidence or finds a real bug.

A1 — water cold-flow for Cd. Cheap, safe, removes the largest single arbitrary multiplier.

B1 + B2 — convergence and uncertainty propagation, so you know what the error bars actually are before committing to hardware.

A3 — measure a and n from your own firings. Per §3.1 this is what actually determines whether the tool is right.

A1 with N2O — discriminate between SPI/HEM/Dyer on real hardware.

Revisit §3.3 and add whichever missing physics the data says matters.